[image: image1.png]J (145800 #29
MATERIAL SAFETY DATA SHEET
NFPA RATING: Health = 0, Flammability = 0, Reactivity = 0
HMIS RATING: Heatth = 0, Flammability = 0, Reactivity = 0

ECTION | ~ IDENTITY AND MANUFACTURER'S INFORMATION (0624A)
- Janufacturer's Name: HILLYARD INDUSTRIES Product Name: PROTECTIVE COATING
Address: 302 North Fourth Street Date Prepared: May 10, 1994
St. Joseph, MO 64502 Prepared by: Regulatory Affairs Department

Emergency Telephone No.: (800)-424-9300 (Only in the event of chemical emergency invoiving a spill, leak, fire, exposure

or accident involving chemicals.) Other Informatlon calls: (816)233-1321(Ext. 303)

SECTION Il - INGREDIENTS/IDENTITY INFORMATION

Components OTHER LIMITS

(Speclfic Chemical Identity: Common CAS# OSHA PEL. ACGIHTLV RECOMMENDED %

Name(s)

Zinc oxide (1) 1314-13-2 5 mga® 5 mg/M° NA. less than

0.1

Water 7732-18-5 none none N.A. -

Acrylate Copolymer 63744-68-3 not not N.A.
established  established

Diethylene glycol monomethy! ether(2) ** 111-77-3 not not N.A. 3-5
established  established

Alkenes, Polymd. emuision 68527-08-2 not not N.A. -
established  established

Tributoxyethyl Phosphate ** 78-51-3 not not N.A. 0.5-15

established  established
(1) Reguiated by OSHA and the foliowing states: FL., IL., LA., MA_, NJ.
{2) Reguiated by the following states: FL., PA., RL.
** This product contains the following chemical subject to reporting requirements of SARA Title Ill, Section 313: Diethylene
~ycol monomethyl ether and Tributoxyethyl Phosphate

SECTION il - PHYSICAL / CHEMICAL CHARACTERISTICS

Bolling Point: 205°F Specific Gravity (Hg0 = 1): 25°C = 1.02 & 399C = 1.02

Vapor Pressure (mm Hg.): 18.0 Percent Volatile by Volume (%): 83.7%

Vapor Density (AIR=1): 1.0 Evaporation Rate {ethy! ether = 1): slower than 1

Solubility In Water: appreciable Appearance and Odor: Creamy, white emuision;
non-objectionable odor.

SECTION 1V -- FIRE AND EXPLOSION HAZARD DATA .
Flash point: None (T.C.C) Flammable Limits: LEL = NA. UEL = N.A.
Extinguishing Media: Foam, Carbon Dioxide, Dry Chemical, Water Fog.
Speclal Fire Fighting Procedures: Wear self-contained breathing apparatus (pressure-demand MSHA/NIOSH
approved or equivalent) and full protective gear.
Unusual Fire and Explosion Hazards: Material may spatter if temperatures exceed boiling point.
Polymer tilm can burn.

SECTION V - PHYSICAL HAZARDS
Stabillty: Stable Conditions to Avold: N/A
Incompatlbility (Materials to Avoid): None known to Hillyard
Hazardous Decomposition Products or Byproducts: None known to Hillyard
Hazardous Polymerization: Will not Occur Condttions to Avold: N/A

SECTION VI - HEALTH HAZARD DATA
Routes of entry: Inhalation? N/A Skin? N/A Ingestion? N/A
HEALTH HAZARDC (1. Acute and 2. Chronic)
1. None known. (According to Primary Skin irritation Test [FHSA], product is not an imitant; Eye Irritation Test
[FHSA], not an irritant.} 2. None known to Hillyard Industries





[image: image2.png]page 2 of #290 PROTECTIVE COATING

HEALTH HAZARD DATA (CONTINUED):
1emical listed as Carclnogen or Potentlai Carcinogen:
wational Toxicology Program = No  1.A.R.C. Monographs = No OSHA = No -
This product has no carcinogens listed by IARC, NTP, NIOSH, OR ACGIH as of this date, greater than or equal 1o 0.1%.
Slgns and Symptoms of Exposure:
None known to Hillyard.
Medical Conditions Generaily Aggravated by Exposure:
None known to Hillyard
Emergency and First Ald Procedures:
If splashed into eyes flush with water for 15 minutes. Flush skin with water. Ingestion: Drink large quantity of water; induce
vomiting; call physician.

SECTION Vil - PRECAUTIONS FCR SAFE HANDLING AND USE

Steps To Be Taken In Case Materlal Is Released Or Spilled: Pick up with mop or wet vac and wash away with water.
Waste Disposal Method: Wash small quantities down sewer with large amount of water. Keep spills and cleaning runoffs
out of municipal sewers and open bodies of water. Large spifls should be picked up with wet vac or absorbent material.
Landfill or incinerate solids. Dispose of liquid per local, state and federal regulations.

Precautlons To Be Taken In Handling And Storing: Freezing of product may rupture container. Product shelf life is best
retained by storage at 45 - 100°F temperature. )

Other Precautions: Inhalation exposure to spray mist should be maintained at less than 10 mgM® per eight hours
exposure (TWA limit estimated for nuisance dust).

SECTION Vit - CONTROL MEASURES:
Resplratory Protection (Specify Type): None required if good ventilation is maintained.

Ventlation:
Local Exhaust = Not required Mechanical (General) = Suggested Speclal = N.A. Other=NA.
Protective Gloves: None required Eye Protectlon: None required

ier Protective Clothing or Equipment: None required.
work / Hyglenic Practices: Do not take intemally; wash contaminated clothing and equipment before reuse.

SECTION IX - TRANSPORTATION INFORMATION:
Applicable regulations: no 49 CFR; no IMCO; no IATA.
Proper shipping name:_Liguid Floor Wax & Pglish ’
UN No.: not applicable Limited Qty.: not applicable ~ Hazard Class: not applicable
Labels required: not required DOT Exception: not applicable
EPA Hazardous waste number / code: not listed

Hazardous waste characteristics: ~
Ignitabllity = not applicable; Corrosivity = not applicable; Reactivity = not applicable.
: DISCLAIMER OF WARRANTIES

NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE,
OR OF ANY NATURE ARE MADE WITH RESPECT TO THE PRODUCT(S) OR INFORMATION CONTAINED IN THIS MATERIAL SAFETY DATA
SHEET.

The information and recommendationa contained in this Material Satoty Data Sheet are supplied pursuant to 29 CFR 1910.1200 of the
Occupational Safety and Health Standards Hazard Communication Rule. All information contained hersin is prossnted in good faith and is
believed to bs appropriats and accurate.

THE BUYER OR USER ASSUMES ALL RISKS ASSOCIATED WITH THE USE, MISUSE OR DISPOSAL OF THIS PRODUCT. THE BUYER OR USER IS
RESPONSIBLE TO COMPLY WITH ALL FEDERAL, STATE OR LOCAL REGULATIONS CONCERNING THE USE, MISUSE OR DISPOSAL OF THESE
PRODUCTS.




