r

Date Issued: 1987-06-17 Supersedes:  1997-06-O3

TEXACO	845/20

MATERIAL SAFETY DATA SHEET

NOTE: Read and understand Material Safety Data Sheet before handling or disposing of product.

1. CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

MATERIAL IDENTITY Product Code and Name: 01854 ATF MERCON/DEXRON III

Chemical Name and/or Family or Description: Transmission Fluid

Manufacturer's Name and Address:

TEXACO LUBRICANTS COMPANY

P.O. Box 4427

Houston, TX 77210-4427

Telephone Numbers:

Transportation Emergency-Company

(914)	831-340O

(800)	424-93OO

(80O)	567-7455

(914)	831-34OO

(914)	838-72O4

(713)	432-3383

(914)	838-7336

(512)	459-6543

(SCO)	782-7852(Option 4)

CHEMTREC (USA) In Canada

Health Emergency       -Company General MSDS Assistance Texaco FaxBack System Technical Information  -Fuels

-Chemical -Lubricant/

Antifreezes/Fuel Additives

(80O) 876-3738

-Solvents

2. COMPOSITION/INFORMATION ON INGREDIENTS

THE CRITERIA FOR LISTING COMPONENTS IN THE COMPOSITION SECTION IS AS FOLLOWS: CARCINOGENS ARE LISTED WHEN PRESENT AT 0.1 % OR GREATER; COMPONENTS WHICH ARE OTHERWISE HAZARDOUS ACCORDING TO OSHA ARE LISTED WHEN PRESENT AT 1.0 X OR GREATER; NON-HAZARDOUS COMPONENTS ARE LISTED AT 3.O % OR GREATER. THIS IS NOT INTENDED TO BE A COMPLETE COMPOSITIONAL DISCLOSURE.  REFER TO SECTION 14 FOR APPLICABLE STATES' RIGHT TO KNOW AND OTHER REGULATORY INFORMATION.

Product and/or Component(s) Carcinogenic According to: OSHA    IARC   NTP    OTHER    NONE

X

Composition: (Sequence Number and Chemical Name)

CAS Number

Seq. Chemical Name

01

Sol vent-dewaxed heavy paraffinlc petroleum distil lates

02	/f Hydrotreated light paraffinlc petroleum

distillate

03	# Hydrotreated light naphthenlc petroleum

distillates

- *   Pol ymethacry late

- *  Boronated hydrocarbyl   sucdnlmide

Range in %

64742-65-0 5O.OO-64.99 64742-55-8 35.0O-49.99 64742-53-6 1.0O-2.99

50867-55-5  1.OO-2.99 1.OO-2.99

PRODUCT IS NON-HAZARDOUS ACCORDING TO OSHA (1910.12OO).

» COMPONENT IS HAZARDOUS ACCORDING TO OSHA.

# COMPONENT, BY DEFINITION, IS CONSIDERED HAZARDOUS ACCORDING TO OSHA

BECAUSE IT CARRIES THE PERMISSIBLE EXPOSURE LIMIT (PEL) FOR MINERAL OIL

MIST.

Exposure Limits referenced by Sequence Number in the Composition Section

Limit

mg/m3 TWA-OSHA (MINERAL OIL MIST) mg/m3 TWA-ACGIH (MINERAL OIL MIST) mg/m3 STEL ACGIH (MINERAL OIL MIST) mg/m3 TWA-OSHA (MINERAL OIL MIST) mg/m3 TWA-ACGIH (MINERAL OIL MIST) mg/m3 STEL ACGIH (MINERAL OIL MIST) mg/m3 TWA-OSHA (MINERAL OIL MIST) mg/m3 TWA-ACGIH (MINERAL OIL MIST) mg/m3 STEL ACGIH (MINERAL OIL MIST)

N.D. - NOT DETERMINED <    - LESS THAN

PAGE :

1

N.A. - NOT APPLICABLE >    - GREATER THAN

N.T. - NOT TESTED

PRODUCT CODE: 01854	Data Issued: 1997-06-17

NAME: ATF MERCON/DEXRON III	Supersedes: 1997-O6-O3

3.	HAZARD IDENTIFICATION""' ~"~~"

EMERGENCY OVERVIEW	~     ~~'	-v

Appearance:

Red 1iquid Odor:

Slight additive odor

WARNING STATEMENT

NONE CONSIDERED NECESSARY

HMZS	NFPA

Health:       1    Reactivity: 0     Health:       1    Reactivity: O Flammabi1ity: 1    Special   : -      Flammabi1ity: 1    Special   : -

POTENTIAL HEALTH EFFECTS

EYE   SKIN   INHALATION   INGESTION
Primary Route of Exposure:  XXX	_

EFFECTS OF OVEREXPOSURE Acute: Eyes:

May cause minimal Irritation, experienced as temporary discomfort.

Skin:

Brief contact may cause slight irritation.  Prolonged contact, as with clothing wetted with material, may cause more severe irritation and discomfort, seen as local redness and swelling.

Other than the potential skin irritation effects noted above, acute (short term) adverse effects are not expected from brief skin contact; see other effects, below, and Section 11 for information regarding potential long term effects.

Inhalation:

Vapors or m1st, in excess of permissible concentrations, or in unusually

high concentrations generated from spraying, heating the material or as	«>

from exposure in poorly ventilated areas or confined spaces, may cause	^

irritation of the nose and throat, headache, nausea, and drowsiness.

Ingest1on:

If more than several mouthfuls are swallowed, abdominal discomfort, nausea, and diarrhea may occur.

Sensltizatlon Properties:

Unknown.

Chronic:

No adverse effects have been documented in humans as a result of chronic exposure.  Section 11 may contain applicable animal data.

Medical Conditions Aggravated by Exposure:	. .

Because of its Irritating properties, repeated skin contact may aggravate an existing dermatitis (skin condition).

Other Remarks:

None

4.	FIRST AID MEASURES      ~*	"~     ""  ~

Eyes:  '    '    ~~         '  ~ '  "  '  '    ~  " "  "  '  "    " Flush eyes with plenty of water for several minutes.  Get medical attention If eye irritation persists.

Skin:

Wash skin with plenty of soap and water for several minutes.  Get medical attention if skin irritation develops or persists.

Ingest1 on:

If more than several mouthfuls of this material are swallowed, give two glasses of water (16 02.).  Get medical attention.

PAGE:

N.D. - NOT DETERMINED       N.A. - NOT APPLICABLE       N.T. - NOT TESTED
<   - LESS THAN	>    - GREATER THAN

PRODUCT CODE: 01854	Date Issued: 1987-O6-17

NAME: ATF MERCON/DEXRON III	Supersedes: 1997-O6-03

4.	FIRST AID MEASURES (CONT)~~   ~      ~~"

Inhalation:

If Irritation, headache, nausea, or drowsiness occurs, remove to fresh air. Get medical attention if breathing becomes difficult or respiratory Irritation persists.

Other Instructions:

Remove and dry-clean or launder clothing soaked or soiled with this material before reuse.  Dry cleaning of contaminated clothing may be more effective than normal laundering.  Inform individuals responsible for cleaning of potential hazards associated with handling contaminated clothing.

5.	FIRE-FIGHTING MEASURES'~~~  ~~~~~"~~

Ignition Temperature - AIT (degrees FT:~

Not determined. Flash Point (degrees F):

> 35O (COC) Flammable Limits (%):

Lower:  Not determined.

Upper:  Not determined.

Recommended Fire Extinguishing Agents And Special Procedures:

Use water spray, dry chemical, foam, or carbon dioxide to extinguish flames.  Use water spray to cool fire-exposed containers.  Water or foam may cause frothing.

Unusual or Explosive Hazards:

None

Extinguishing Media Which Must Not Be Used:

Not determined.

Special Protective Equipment for Firefighters:

Wear full protective clothing and positive pressure breathing apparatus.

6.	ACCIDENTAL RELEASE MEASURES (Transportation Spills:CHEMTREC (800)424-93OO)

Procedures in Case of Accidental Release, Breakage or Leakage:~ Ventilate area.  Avoid breathing vapor.  Wear appropriate personal protective equipment, including appropriate respiratory protection. Contain spill 1f possible.  Wipe up or absorb on suitable material and shovel up. Prevent entry into sewers and waterways.  Avoid contact with skin, eyes or clothing.

If more than    1,333,333 pounds of product 1s spilled, then report spill according to SARA 304 and/or CERCLA 1O2(a) requirements, unless product qualifies for the petroleum exemption (CERCLA Section 101(14)).

7.	HANDLING AND STORAGE~~~~~~~

Precautions to be Taken in	"~  ~   ~~~ ~~~

Handling: Minimum feasible handling temperatures should be maintained.

Storage:

Periods of exposure to high temperatures should be minimized. Water contamination should be avoided.

8.	EXPOSURE CONTROLS/PERSONAL PROTECTION~~~~ ~~

Protective Equipment (Type)	~~  ~       ~~ ~~~~~

Eye/Face Protection:

Safety glasses, chemical type goggles, or face shield recommended to prevent eye contact.

Skin Protection:

Workers should wash exposed skin several times dally with soap and water. Soiled work clothing should be laundered or dry-cleaned.

PAGE:

N.D. - NOT DETERMINED       N.A. - NOT APPLICABLE       N.T. - NOT TESTED
<    - LESS THAN	>    - GREATER THAN

PRODUCT CODE: 01854	Data Issued: 1997-O6-17

NAME: ATF MERCON/DEXRON III	Supersedes: 1997-O6-O3

a. EXPOSURE CONTROLS/PERSONAL PROTECTION (CONT)"~"

Respiratory Protection:

Airborne concentrations should be kept to lowest levels possible.  If vapor, mist or dust 1s generated and the occupational exposure limit of the product, or any component of the product, is exceeded, use appropriate NIOSH or MSHA approved air purifying or air supplied respirator after determining the airborne concentration of the contaminant. A1r supplied respirators should always be worn when airborne concentration of the contaminant or oxygen content 1s unknown.

Ventilation:

Adequate to meet component occupational exposure limits (see Section 2).

Exposure Limit for Total Product:

None established for product; refer to Section 2 for component exposure limits.

9. PHYSICAL AND CHEMICAL PROPERTIES~~~~

Appearance:	"    ~   ~        '

Red 11qu i d Odor:

Slight additive odor

Boiling Point (degrees F):

Not determined.

Melting/Freezing point (degrees F): Not applicable.

Specific Gravity (water=1): .869

pH of undiluted product:

Not applicable.

Vapor Pressure:

Not determined.

Viscosity:

> 2O cSt at 40.0 C

VOC Content: 38 g/L

Vapor Density (a1r=1):

Not determined.

Solubility in Water (%): Not determined.

Other: None
10. STABILITY AND REACTIVITY	~~    ~~	"~~~

This Material Reacts Violently With:"~~~~~~"

(If Others is checked below, see comments for details) Air  Water  Heat  Strong Oxidlzers  Others  None of These

X Comments:

None

Products Evolved When Subjected to Heat or Combustion:

Toxic levels of carbon monoxide, carbon dioxide. Irritating aldehydes and ketones.

Hazardous Polymerizations: DO NOT OCCUR

PAGE:

N.D. - NOT DETERMINED      N.A. - NOT APPLICABLE       N.T. - NOT TESTED
<    - LESS THAN	>    - GREATER THAN

PRODUCT CODE: 01854	Date Issued: 1897-06-17

NAME: ATF MERCON/DEXRON IZI      ,	Supersedes: 19S7-O6-O3

11.	TOXICOLOGICAL INFORMATION

TOXICOLOGICAL INFORMATION(ANIMAL TOXICITY DATA) Median Lethal Dose Oral:

LD50 Believed to be > 5. CO g/kg (rat) practically non-toxic Inhalation:

Not determined. Dermal:

LD5O Believed to be > 2.OO g/kg (rabbit) practically non-toxic Irritation Index, Estimation of Irritation (Species) Skin:

(Draize) Believed to be > .50 - 3.OO /8.0 (rabbit) slightly irritating Eyes:

(Draize) Believed to be < 15.OO /11O (rabbit) no appreciable effect Sensitization:

Not determined. Other:

Some low viscosity naphthenic oils have caused skin irritation and skin tumors in laboratory animals when repeatedly applied and left in place between applications.  The significance of this effect to humans has not been establ11 shed.

12.	DISPOSAL CONSIDERATIONS

Waste Disposal Methods

This product has been evaluated for RCRA characteristics and does not meet the criteria of a hazardous waste 1f discarded in its purchased form. Under RCRA, it is the responsibility of the user of the product to determine at the time of disposal, whether the product meets RCRA criteria for hazardous waste.  This is because product uses, transformations, mixtures, processes, etc. may render the resulting materials hazardous.

Remarks

None

13.	TRANSPORT INFORMATION

Transportat i on DOT: Proper Shipping Name:

Not regulated

IMDG: Proper Shipping Name:

Not regulated

ICAO: Proper Shipping Name:

Not regulated

TDG: Proper Shipping Name:

Not regulated

14. REGULATORY INFORMATION

Federal Regulations: SARA Title III: Section 3O2/304 Extremely Hazardous Substances

Seq. Chemical Name	 CAS Number	 Range in %

O1   Hydrochloric acid	7647-O1-0    O.OO525

Section 3O2/3O4 Extremely Hazardous Substances (CONT)

Seq. TPQ	 RQ

01         50O       5OOO

Section 311 Hazardous Categorization:

Acute  Chronic  F1re  Pressure  Reactive  N/A

_        ..	X

PAGE:

N.D. - NOT DETERMINED       N.A. - NOT APPLICABLE       N.T. - NOT TESTED
<    - LESS THAN	>    - GREATER THAN

PRODUCT  CODE:   01354	Data   Issued:    1997-06-17

NAME:   ATr   MERCON/DEXRON   III	Supersedes:    1997-06-03

14.	REGULATORY   INFORMATION   (COST)	~—-—	-	-         -

Section 313 Toxic Chemical

Chem 1 ca 1 Name	 CAS Number	 Concentration

None

CERCLA 102(a)/DOT Hazardous Substances: (+ indicates DOT Hazardous Substance)

Seq. Chemical Name	 CAS Number    Range 1n %

01+  Xylene	133O-20-7    O.OO75

02+ Propenolc acid, ethyl ester	14O-88-5   O.OO525

03+  Hydrochloric acid	7S47-O1-O   O.OO525

04+  Ethyl benzene	1OO-41-4   O.OO1675

CERCLA/DOT Hazardous Substances (Sequence Numbers and RQ's):

Seq. KQ

01+ 1OO 02+ 10OO 03+ 5OOO 04+ 1OOO

TSCA Inventory Status:

This product, or its components, are listed on or are exempt from the Toxic Substance Control Act (TSCA) Chemical Substance Inventory.

Other:

Texaco has performed a risk assessment on this product which indicates that no California Proposition 65 warnings are required.

State Regulations: California Proposition 85:

The following detectable components of this product are substances, or belong to classes of substances, known to the State of California to cause cancer and/or reproductive toxicity.

Chemical Name	 CAS Number

Propenoic acid, ethyl ester	14O-88-5

International Regulations:

WHMIS Classification:

Not control led

Canada Inventory Status:

This product, or its components, are listed on or are exempt from the Canadian Domestic Substance List (DSL).

EINECS Inventory Status:

Not determined.

Australia Inventory Status:

Not determined.

Japan Inventory Status:

Not determined.

15.	ENVIRONMENTAL. INFORMATION   ~ ~	~~"	~ ~~~"

Aquatic Toxicity:      ^      ~   " ~~    — —    — Not determined.

Mobility:

Not determined.

Persistence and Biodegradabi1ity:

Not determined.

Potential to Bloaccumulate:

Not evaluated.

Remarks:

Not evaluated.

— 18. OTHER INFORMATION      ~ ~   '      "	"   ~    ~

None
	 PAGE:   8

N.D. - NOT DETERMINED       N.A. - NOT APPLICABLE       N.T. - NOT TESTED
<    - LESS THAN	>    - GREATER THAN

PRODUCT CODE: 01854

NAME: ATF MERCON/DEXRON III

Date Issued: 1887-06-17 Supersedes: 1987-O6-03

16. OTHER INFORMATION (CONT)

THE INFORMATION CONTAINED HEREIN IS BELIEVED TO BE ACCURATE.  IT IS PROVIDED INDEPENDENTLY OF ANY SALE OF THE PRODUCT FOR PURPOSE OF HAZARD COMMUNICATION AS PART OF TEXACO'S PRODUCT SAFETY PROGRAM.  IT IS NOT INTENDED TO CONSTITUTE PERFORMANCE INFORMATION CONCERNING THE PRODUCT.  NO EXPRESS WARRANTY. OR IMPLIED WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE IS MADE WITH RESPECT TO THE PRODUCT OR THE INFORMATION CONTAINED HEREIN.  DATA SHEETS ARE AVAILABLE FOR ALL TEXACO PRODUCTS.  YOU ARE URGED TO OBTAIN DATA SHEETS FOR ALL TEXACO PRODUCTS YOU BUY, PROCESS, USE OR DISTRIBUTE AND YOU ARE ENCOURAGED AND REQUESTED TO ADVISE THOSE WHO MAY COME IN CONTACT WITH SUCH PRODUCTS OF THE INFORMATION CONTAINED HEREIN.

TO DETERMINE APPLICABILITY OR EFFECT OF ANY LAW OR REGULATION WITH RESPECT TO THE PRODUCT, USER SHOULD CONSULT HIS LEGAL ADVISOR OR THE APPROPRIATE GOVERNMENT AGENCY.  TEXACO DOES NOT UNDERTAKE TO FURNISH ADVICE ON SUCH MATTERS.

Since the last mailing for this customer code, the following sections have been revised:

e, 14,16,17.

Date: 1997-06-17    _ Date printed: 1997-O6-17

New

Revised, Supersedes: 1997-O6-03

Inquiries regarding MSDS should be directed to: Texaco Inc.

Manager, Product Safety P.O. Box 5O9 Beacon, N.Y.  125O8

PLEASE SEE NEXT PAGE FOR PRODUCT LABEL

N.D. - NOT DETERMINED <    - LESS THAN

PAGE:   7

N.A. - NOT APPLICABLE >    - GREATER THAN

N.T. - NOT TESTED

PRODUCT CODE: 01854	Date Issued: 1997-O6-17

NAME: ATF MERCON/DEXRON III	Supersedes: 1997-06-03

17. PRODUCT LABEL"—-Label Date: 1997-06-17

READ AND UNDERSTAND MATERIAL SAFETY DATA SHEET BEFORE HANDLING OR DISPOSING OF PRODUCT.  THIS LABEL COMPLIES WITH THE REQUIREMENTS OF THE OSHA HAZARD COMMUNICATION STANDARD (29 CFR 191O.12OO) FOR USE IN THE WORKPLACE.  THIS LABEL IS NOT INTENDED TO BE USED WITH PACKAGING INTENDED FOR SALE TO CONSUMERS AND MAY NOT CONFORM WITH THE REQUIREMENTS OF THE CONSUMER PRODUCT SAFETY ACT OR OTHER RELATED REGULATORY REQUIREMENTS.

01854 ATF MERCON/DEXRON III

WARNING STATEMENT NONE CONSIDERED NECESSARY

PRECAUTIONARY MEASURES

-Avoid prolonged breathing of vapor, mist, or gas. -Workers should wash exposed skih several times daily with soap and water.

FIRST AID Eye Contact:

Flush eyes with plenty of water for several minutes.  Get medical attention if eye Irritation persists. Skin Contact:

Wash skin with plenty of soap and water for several minutes.  Get medical attention if skin irritation develops or persists. Ingestion:

If more than several mouthfuls of this material are swallowed, give two glasses of water (16 oz.) .  Get medical attention. Inhalation:

If irritation, headache, nausea, or drowsiness occurs, remove to fresh air. Get medical attention if breathing becomes difficult or respiratory Irritation persists. Note to Physician: None

FIRE

In case of fire, use water spray, dry chemical, foam or carbon dioxide. Water may cause frothing. Use water spray to cool fire-exposed containers.

If more than    1,333,333 pounds of product is spilled, then report spill according to SARA 304 and/or CERCLA 1O2(a) requirements, unless product qualifies for the petroleum exemption (CERCLA Section 101(14)).

Chemical Name	  CAS Number  _  Range i n %

#	Solvent-dewaxed heavy paraffinic petroleum      S4742-65-0 5O.OO-S4.99

distillates

If   Hydrotreated light paraffinic petroleum         64742-55-3 35.0O-49.99 distil late

#	Hydrotreated light naphthenic petroleum         64742-53-6  1.0O-2.99

distillates

- Polymethacry late	5O867-55-5  1.OO-2.99

- Boronated hydrocarbyl succinimlde	1.0O-2.99

PRODUCT IS NON-HAZARDOUS ACCORDING TO OSHA (191O.12OO).

*	COMPONENT IS HAZARDOUS ACCORDING TO OSHA.

#	COMPONENT, BY DEFINITION, IS CONSIDERED HAZARDOUS ACCORDING TO OSHA
BECAUSE IT CARRIES THE PERMISSIBLE EXPOSURE LIMIT (PEL) FOR MINERAL OIL
MIST.

«

Pennsylvania Special Hazardous Substance(s)    CAS Number     Range in % None

HMIS	NFPA

Health:	1	Reactivity:   0	Health:	1	Reactivity:   O

Flammabi11ty:    1	Special         :   -	Flammabi11ty:    1	Special         :   -

Transportation DOT: Proper Shipping Name:

Not  regulated

PAGE:   3

N.D. - NOT DETERMINED      N.A. - NOT APPLICABLE      N.T. - NOT TESTED
<    - LESS THAN	>    - GREATER THAN

PRODUCT CODE: 01854	Date Issued: 1987-O6-17

NAME: ATF MERCON/DEXRON III	Supersedes: 19S7-O6-03

17. PRODUCT LABEL (CONT)Label Date: 1997-06-17

CAUTION:  Misuse of empty containers can be hazardous.  Empty containers can be hazardous if used to store toxic, flammable, or reactive materials.  Cutting or welding of empty containers might cause fire, explosion or toxic fumes from residues.  Do not pressurize or expose to open flame or heat.  Keep container closed and drum bungs in place.

Manufacturer's Name and Address:

TEXACO LUBRICANTS COMPANY

P.O. Box 4427

Houston,   TX   77210-4427

TRANSPORTATION  EMERGENCY     Company:         (914)   831-34OO

CHEMTREC:       (8OO)   424-93OO

HEALTH   EMERGENCY	Company:         (914)   831-34OO